# Class Diagram Documentation – SRT Anomaly Detection

## Purpose

The class diagram represents the logical structure of the anomaly detection system for the Sardinia Radio Telescope (SRT). It illustrates:

- The main classes within the system.

- Their respective responsibilities, including attributes and methods.

- The collaboration along the pieline: data loading → density-based filtering → score calculation by frequency and similarity filters → ranking → visualization.

## Class Description

### Candidate

Represents a single spectral candidate extracted from SRT observations.

Attributes:

- id : String – Unique identifier.

- frequency_hz : float – Center frequency of the signal in Hz.

- drift_hz_s : float – Drift rate in Hz/s.

- cadence : array – Cadence tensor, typically of shape (6, H, W), containing the 6 ON/OFF panels.

- source_path : Path – Path to the source file for traceability.

- category : int - Diagnostic helper storing the argmax category assigned by the density filter.

- density_score : float – Score calculated by the DensityFilter.

- frequency_score : float – Score calculated by the FrequencyFilter.

- similarity_score : float – Score calculated by the SimilarityFilter.

- target : str – indentifier of the observed source used to stratify candidates during the ranking phase.

- band : str – frequency band label (C, K) derived from the candidate central frequency and used for band specific ranking.

Methods:

- to_summary(): Returns a dictionary containing metadata, all computed scores, and the source path.

- set_category(value) : Sets the diagnostic density argmax category.

- set_density_score(value): Sets the density score.

- set_frequency_score(value): Sets the frequency score.

- set_similarity_score(value): Sets the similarity score.

### Dataset

Handles data management and preprocessing.

Responsibilities:

- Loading synthetic cadence tensors (.npy) and mapping them to metadata from cadences_log.csv.

- Processing real PNG waterfall plots by parsing filenames, applying notch filter masks, and converting them into standardized Candidate objects.

- Performing signal cleaning, including time-normalization, DC spike removal, and bandpass correction.

Attributes:

- png_dir: Root directory containing the real PNG waterfall plots.

- rx: Regex for parsing metadata from filenames (target, drift rate, frequency, etc.).

Methods:

- load_simulated_cadences(cadences_dir): Reads simulated files.

- load(png_dir): Loads PNG files.

- crop_box(w, h): Crops images to remove margins.

- preprocessing_spectrogram(data): Preprocesses the signal with time normalization, DC spike removal, and bandpass correction using B-splines.

### IFilter <<interface>>

Common interface for all filtering modules.

Responsibilities:

Defining the contract for model training and candidate scoring.

Managing logic for saving and loading models from disk.

Attributes:

- name: Name of the filter.

- random_state: Seed for reproducibility.

Methods:

- fit(data): Trains the filter on provided data.

- calculate(candidate): Calculates a score for a Candidate.

- save_models() / load_models(): Model persistence management.

### DensityFilter

Density-based filter using UMAP + KDE on cross-correlation features.

Responsibilities:

- Training UMAP and KDE models using simulated cadences.

- Evaluating if a real candidate is rare in the UMAP space.

- Filtering out RFI candidates (not used for the final ranking).

Attributes:

- umap_model_path, kde_models_dir, cache_Z_train, cache_y_train, cache_Z_val, cache_y_val, cache_probs: Model storage paths.

- n_neighbors, min_dist, metric, n_components: UMAP hyperparameters.

- threshold: float – The log-density threshold (default: 0.0618) used to reject candidates.

- kde_bandwidth, kde_kernel: KDE parameters.

- kdes: Dictionary of KDE models for each simulation category.

Methods:

- derive_category(meta): Maps simulation metadata to the internal category index (0-63).

- validate_on_val(Z_val, y_val): Runs validation diagnostics.

### FrequencyFilter

Filter based on frequency distribution using GMM ensembles per band.

Responsibilities:

- Modeling frequency distributions for bands L, S, and 10cm.

- Assigning higher scores to candidates in rarer frequency regions.

Attributes:

- gmm_model_path: File containing the GMM ensembles.

- components: GMM components per band.

- n_bags, bag_size: Bagging parameters.

- gmm_ensembles: List of GMMs for each band.

- raw_min / raw_max: float – Global values used to linearly scale the raw anomaly scores into a standardized [0, 1] range.

Methods:

- extract_band(candidate): A static helper method that assigns a candidate to L, S, or 10cm based on its central frequency (Hz).

- raw_density_score(candidate): An internal method that calculates the raw anomaly value defined as 1 / mean\_pdf across the ensemble.

- compute_scaling(band_to_candidates): An internal routine that determines the minimum and maximum raw scores to calibrate the [0, 1] normalization.

### SimilarityFilter

Filter based on the similarity between ON/OFF panels.

Responsibilities:

- Reducing panel resolution from (16, 80) to (8, 16) using block averaging to focus on structural features.

- Embedding downsampled panels into a low-dimensional UMAP space to calculate geometric distances between observation slots.

- Computing a similarity ratio where higher values indicate that ON panels are mutually similar and distinct from OFF panels.

- Normalizing raw distance ratios into a standardized [0, 1] score based on training data statistics.

Attributes:

- umap_model_path: Path – The file system path used to persist the trained UMAP model.

- umap: UMAP – The manifold learning instance used to transform panel pixels into embeddings.

- n_neighbors, _min_dist, _metric, _n_components: UMAP hyperparameters used to configure the embedding space.

- raw_min / _raw_max: float | None – Global range of raw similarity ratios used for linear scaling.

- freq_downsample / _time_downsample: int – Constant factors (5 and 2) used to reduce panel dimensions before embedding.

- on_indices / _off_indices: List[int] – Fixed indices (ON: [0, 2, 4], OFF: [1, 3, 5]) defining the observation pattern.

Methods:

- compute_scaling(Candidate) - Internal routine that determines the global minimum and maximum range of raw scores to calibrate the standardized [0, 1] normalization.

- raw_similarity_score(candidate): Internal method that calculates the raw ratio defined as mean_dist(ON-OFF) / mean_dist}(ON-ON) + epsilon).

- downsample_cadence(cadence): Internal routine that performs block-averaging to downsample the (6, 16, 80) input tensor to (6, 8, 16).

### Ranker

Handles the prioritization and sampling of scored candidates following the paper logic.

Responsibilities:

- Selecting top candidates based on frequency score percentile.

- Selecting top candidates based on similarity score percentile.

- Selecting a combined Top-K using equal weighting of scores.

- Generating a random control sample for validation.

Attributes:

- top_percentile_freq : float — percentile for frequency-based selection

- top_percentile_sim : float — percentile for similarity-based selection

- top_k : int — number of candidates selected in combined ranking

- control_k : int — size of the random control sample

Methods:

- top_percentile_per_group(candidates, score_attr, percentile) - selects the top percentile per (target, band) for a given score.

- top_k_combined(candidates) - returns the Top-K candidates using equal weighting of frequency and similarity scores.

- random_control(candidates) - extracts a reproducible random control sample.

- build_samples(candidates) - builds and returns all ranking samples defined by the pipeline.

### Visualizer

Manages the generation of diagnostic and results figures.

Responsibilities:

- Creating visual representations of candidates (6-panel cadences).

- Producing diagnostic plots for the models (embeddings and score distributions).

Attributes:

- output_dir: Path for saving figures.

Methods:

- plot_umap_embedding(Z, labels, title, filename, on_cat, off_cat, kde_on, kde_off): Generates a 2D UMAP scatter plot featuring marginal histograms for X and Y coordinates. It optionally overlays KDE contour lines for specific target categories.

- plot_density_histogram(densities, threshold, filename): Creates a histogram of the density probabilities on a log scale, including a vertical line representing the rejection threshold.

## Main Relationships

Inheritance: IFilter is implemented by DensityFilter, FrequencyFilter, and SimilarityFilter.

Dependency: All filters, the Ranker, and the Visualizer depend on the Candidate class to access data and store scores.

Creation: The Dataset class is responsible for instantiating Candidate objects.